LAMBERT_1

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

LAMBERT_1 is a C language code component that provides conversions between Geodetic coordinates (latitude and longitude) and Lambert Conformal Conic projection coordinates (easting and northing) with one standard parallel.

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of LAMBERT_1 is to provide a reusable component which supports the following coordinate conversions:

- Geodetic coordinates (latitude and longitude in radians) to Lambert Conformal Conic projection coordinates (easting and northing in meters) with one standard parallel,

- Lambert Conformal Conic projection coordinates (easting and northing in meters) with one standard parallel to Geodetic coordinates (latitude and longitude in radians).

A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a):	Radius (in meters) at the equator, and

- Flattening (f):	Ratio of the difference between the semi-major axis and polar radius of the Earth to its semi-major axis.

A particular variation of the Lambert Conformal Conic projection with one standard parallel is specified in terms of the following parameters:

- Central Meridian – Longitude (in radians) at the origin of the projection,

- Origin Latitude  – Latitude (in radians) at the origin of the projection,

- False Easting – A coordinate value (in meters) assigned to the central meridian of the projection to avoid the inconvenience of using negative coordinates,

- False Northing – A coordinate value (in meters) assigned to the origin latitude of the projection to avoid the inconvenience of using negative coordinates, and

- Scale Factor – A multiplier for reducing a distance in projected coordinates to the actual distance along the origin latitude.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_Lambert_1_Parameters – This function sets the ellipsoid parameters and Lambert Conformal Conic projection parameters for the particular variation of the Lambert Conformal Conic projection that is to be used in subsequent coordinate conversion operations.

Get_Lambert_1_Parameters – This function returns the current values of the ellipsoid parameters and Lambert Conformal Conic projection parameters.

Convert_Geodetic_To_Lambert_1 – This function converts the specified geodetic coordinates (latitude and longitude in radians) to Lambert Conformal Conic projection coordinates (easting and northing in meters) using the current ellipsoid parameters and Lambert Conformal Conic projection parameters.

Convert_Lambert_1_To_Geodetic – This function converts the specified Lambert Conformal Conic projection coordinates (easting and northing in meters) to geodetic coordinates (latitude and longitude in radians) using the current ellipsoid parameters and Lambert Conformal Conic projection parameters.

# EXAMPLE ApplicationS:

The following example illustrates how LAMBERT_1 can be used to convert Geodetic coordinates to Lambert Conformal Conic projection coordinates and back again:

## Function Call:

status = Set_Lambert_1_Parameters (a, b, Origin_Latitude, Central_Meridian, False_Easting, False_Northing, Scale_Factor)

## Inputs:

- a	6378137.0

- f	1 / 298.257223563

- Origin_Latitude	60.0

- Central_Meridian	0.0

- False_Easting	0.0

- False_Northing	0.0

- Scale_Factor	1.0

## Function Call:

status = Convert_Geodetic_To_Lambert_1 (Latitude, Longitude, Easting, Northing)

## Inputs:

- Latitude:	35.0

- Longitude:	75.0

## Outputs:

- Easting:	5934382

- Northing:	918384

## Function Call:

status = Convert_Lambert_1_To_Geodetic (Easting, Northing, Latitude, Longitude)

## Inputs:

- Easting:	5934382

- Northing:	918384

## Outputs:

- Latitude:	35.0

- Longitude:	75.0